Final Year Project Technical Report

BSc (Hons) Computing with Computer Games Development

How effectively can Bluetooth and GPS work together to provide seamless location tagging?

Student Name: Dale Nesbitt

Student ID: L00106269

Declaration

- I have accurately identified and included the sources of all facts, ideas, opinions, and viewpoints from others in the assignment references. All direct quotations, paraphrasing, and discussions of ideas from books, journal articles, internet sources, course materials, or any other sources used are properly acknowledged and cited in the assignment references.

- I have not used unauthorised artificial intelligence tools or aids.

- I understand and am compliant with ATU's policy and procedures regarding Academic Integrity and I am aware of the consequences of any violations.

- I have followed the referencing guidelines recommended in the assignment instructions and / or programme documentation.

- I understand that not adhering to the word limits on each section will result in penalties.

- By signing this form or by submitting material online I confirm that this assignment, or any part of it, has not been previously submitted by me or any other person for assessment on this or any other programme of study.

- By signing this form or by submitting material for assessment online I confirm that I have read and understood AQAE022 Academic Integrity Policy.

I confirm I agree to this declaration. ☒

Print Signature: _______________________________

Date: 01/11/2025

Acknowledgements

I would like to thank…

Table of Contents

1.	Project Overview	1

1.1.	Purpose	1

1.2.	Objectives	1

1.2.1.	One liner	1

1.2.2.	Sub-Item	1

1.3.	Project Scope	1

1.4.	Definitions and Acronyms	2

2.	Requirements	3

2.1.	Functional Requirements	3

2.2.	Non-Functional Requirements	3

2.3.	Table Section	3

2.4.	Code and Formulae	4

References	i

Appendix A: Report Structure Example	ii

Appendix B:  Code Listing / GitHub Project Link	iv

Table of Figures

Figure 1 Atlantic Technological University Donegal Logo	2

Figure 2 ATU Donegal, Letterkenny	2

Glossary of Terms

| Term | Definition |
| --- | --- |
| BLE (Bluetooth Low Energy) | A type of wireless communication designed for short-range, low-power data exchange that operates in the 2.4 GHz band. Commonly used for proximity sensing, wearables, and IoT devices where battery efficiency is important. (Fürst et al., 2018) |
| GPS (Global Positioning System) | A satellite-based navigation system that provides absolute location and accurate time information anywhere on Earth. Alternative versions such as GLONASS (Russian Federation), BeiDou (China) and Galileo (European Union) also exist, but they all perform the same function. (Rutkowski, 2022) |
| RSSI (Received Signal Strength Indicator) | A measurement of how strong a received radio signal is. In BLE systems, RSSI is used to estimate how near or far a device is. (Chai, An and Du, 2016) |
| ESP32 | A microcontroller unit with integrated Wi-Fi and Bluetooth connectivity for a wide-range of applications. (ESP32 Wi-Fi & Bluetooth SoC ¦ Espressif Systems, no date) |
| NEO-8M | A small GPS receiver module that outputs location data using standard protocols like NMEA. It connects to microcontrollers via UART and provides reliable GPS fixes. It is a specific type of GNSS module. (NEO-M8 series, 2015) |
| GNSS | A Global Navigation Satellite System (GNSS) module is an electronic device that receives signals from satellite constellations such as GPS, GLONASS, Galileo, or BeiDou to calculate its geographic position in real time. (Rutkowski, 2022) |
| NMEA (National Marine Electronics Association Format) | A standardized text-based format used by GPS modules to transmit location, time, and status data. Messages are structured as comma-separated sentences starting with a “$” symbol. (Gakstatter, 2015) |
| ms (milliseconds) | A unit of time equal to one-thousandth of a second (0.001 s). (Merriam-Webster, 2025) |
| Hz (Hertz) | A unit of frequency meaning equal to one cycle per second. (Merriam-Webster, 2025) |
| GGA | An NMEA sentence that provides key GPS fix information, including latitude, longitude, fix quality, and number of satellites. Used in this project to validate the reliability of each GPS reading. |

# Project Overview

## Purpose

I will investigate how Bluetooth Low Energy (BLE) and Global Positioning System (GPS) technologies can be combined to create a hybrid ‘sensing’ model capable of supporting context-aware IoT applications to determine location. My primary goal will be to design and evaluate a BLE/GPS ‘handshake mechanism’, where BLE proximity events and GPS location updates interact to determine meaningful system states. I will focus on the technical synchronisation between BLE’s short-range RSSI-based detection and GPS’s global positional data, examining how these two sensing modalities can be used together to deliver reliable location tagging on a low-power microcontroller platform. I am hoping to provide a clear understanding of the limitations, timing behaviours, and accuracy characteristics involved in hybrid BLE/GPS sensing.

## Objectives

### 1.2.1 	BLE Proximity Detection

Implement a BLE 4.2 scanning system on the ESP32 that detects a predefined BLE tag, retrieves its RSSI values, and classifies proximity states (e.g., near, far, lost). I chose BLE 4.2 because it is already integrated into the ESP32 hardware (Espressif, 2025). This means no additional antennas, modules, or power overhead are required. It is more than capable of delivering the short-range proximity readings I need, and it maintains low power consumption while still offering stable scanning performance. The system must identify ‘tag-loss’ events within a scanning interval of <500 ms (Bluetooth SIG, 2014) and operate reliably across typical indoor and outdoor environments.

### 1.2.2	 GPS Location Detection/Acquisition

Integrate the NEO-8M GPS module to obtain real-time coordinates. GPS fix-quality and satellite-count data will be used to validate readings and prevent false classifications. (NEO-M8 series, 2015)

### 1.2.3	 BLE/GPS Handshake

Design a deterministic “handshake” workflow where BLE proximity loss triggers GPS location verification, and GPS updates influence BLE scanning behaviour. The handshake must define clear timing rules, thresholds, and decision logic for transitions between contextual system states (e.g., departure detected, geofence entry, false BLE loss).

### 1.2.4	 Event Logging

Develop a local logging system using internal flash of the ESP32 or MicroSD storage to record BLE RSSI readings, GPS coordinates, fix quality, handshake outcomes, and system states.

### 1.2.5 	System Feedback

Implement a visual or auditory output mechanism that reflects handshake-determined system states such as “BLE tag lost”, “geofence entered”, or “GPS invalid”. Feedback must be driven solely by validated BLE/GPS transitions.

### 1.2.6 Performance Evaluation

Evaluate the hybrid sensing system by analysing BLE range variability, GPS lock time, handshake timing behaviour, and false-positive/false-negative rates using the collected log data. This evaluation will determine the reliability and limitations of the coordinated BLE/GPS sensing model.

## Project Scope

In Scope:

- Development of a hybrid BLE/GPS sensing system on the ESP32 platform.

- Implementation of BLE scanning, GPS location/data acquisitions, handshake logic and event logging

- Technical evaluation of synchronisation timing, accuracy, and system reliability.

Out of Scope:

- Cloud integration, remote databases, or mobile app development.

- Behavioural user testing or human-factor evaluation.

- AI-based modelling, multi-user capabilities, or commercial product design.

- Environmental/social impact analysis.

Assumptions:

- Adequate GPS satellite visibility during operation.

- BLE tags comply with Bluetooth 4.2 advertising behaviour. Components are often branded as being compliant when they, in fact, are not.

- Local power (vehicle or battery) is available.

- Environmental conditions permit stable BLE and GPS communication.

Dependencies:

- ESP32 BLE and GPS software libraries.

- ESP32’s BLE version correctly branded.

- Reliable NEO-8M GPS operation and NMEA sentence output.

# Requirements

## Functional Requirements

BLE Scanning

- The system shall scan for a specific BLE tag at an interval of <500 ms.

- The system shall retrieve RSSI values and classify proximity states.

Proximity Loss Detection

- The system shall determine tag loss after X consecutive missed scan cycles. (X shall be determined at a later date).

GPS Coordinate Acquisition

- The system shall retrieve GPS coordinates at a minimum of 1 Hz. (NEO-M8 series, 2015)

- The system shall validate readings using fix quality and satellite count. (Active Research Ltd., 2015)

Geofence Detection

- The system shall detect when coordinates enter or exit predefined geofences.

BLE/GPS Handshake

- The system shall trigger GPS checks when BLE proximity loss occurs.

- The handshake shall classify transitions such as departure, arrival, and false loss.

Event Logging

- Logs shall include timestamps, RSSI, GPS data, handshake results, and state transitions.

Feedback Output

- The system shall output visual/auditory indicators for state changes.

## Non-Functional Requirements

Performance

- The system shall maintain a BLE scan interval of <500 ms and retrieve GPS coordinates at a minimum frequency of 1 Hz to ensure a responsive BLE/GPS handshake execution.

Accuracy

- The system shall achieve a typical GPS positional accuracy of 2.5 - 5 metres in open-sky conditions (NEO-M8 series, 2015).

Reliability

- The system shall operate continuously without unexpected interruptions to BLE scanning or GPS data acquisition, and it shall recover quickly from temporary GPS signal loss.

Robustness to Environmental Factors (Ireland)

- The system shall remain functional under moderate environmental interference such as BLE multipath effects, signal attenuation, and varying satellite visibility due to adverse weather conditions.

Power Efficiency

- The system should be capable of operating on portable power for at least 2 hours, with BLE scan intervals and GPS polling rates chosen to balance responsiveness and power usage.

Security

- The system should avoid the storage of personal or identifying data and should use BLE advertising mode without pairing to reduce security risks.

# System Architecture & Design

## High-Level Architecture Overview

My system will be built around a hybrid sensing approach that combines BLE for short-range proximity detection with GPS for absolute, accurate positional awareness. The ESP32 microcontroller acts as the central controller, coordinating the built-in BLE 4.2 and GPS subsystems and executing the handshake logic that determines the system’s context. The overall architecture is deliberately simple at this stage, as the detailed implementation will be carried out next semester.

The key components of the architecture are:

- BLE subsystem, which handles scanning for a specific tag and reading RSSI values; BLE scans occur at <500 ms intervals

- The GPS subsystem receives positional data from the NEO-8M module using NMEA sentences (Active Research Ltd., 2015) and updates at approximately 1 Hz.

- Handshake engine, which evaluates BLE and GPS data together to determine system state transitions.

- Dual sensor layer, which compares proximity information with location information.

- Event logger, which records system events and environmental data to either SD card or internal flash.

This high-level structure outlines how the major parts of the system are intended to work together, without going into implementation detail. This will be investigated and fine-tuned next semester.

## Component Descriptions

Bluetooth Low Energy Module

BLE is used to detect the presence or absence of a designated tag by scanning for its advertising packets and reading the associated RSSI values. The ESP32’s built-in BLE 4.2 support makes it suitable for short-range proximity detection without requiring additional hardware. BLE range and signal strength vary depending on the environment, and RSSI readings may fluctuate due to interference or obstacles. The scan interval determines how quickly the system can detect proximity changes and affects overall power usage.

GPS Module (NEO-8M)

The NEO-8M GPS module provides global position data at an update rate of approximately one hertz, with typical accuracy of around 2.5 metres under good satellite visibility. Cold-start times range from roughly twenty to thirty seconds. The module outputs NMEA sentences such as GGA, which provide coordinates, fix quality, and satellite count. GPS performance reduces significantly indoors or in areas with obstructed sky view.

## BLE/GPS Handshake Process

The handshake between BLE and GPS is the central behaviour that I will investigate. When the BLE subsystem detects that the tag’s RSSI has fallen below a threshold or that the tag has not appeared for a predefined number of scans, it flags a potential loss event. This triggers the handshake engine to request an updated GPS reading.

The GPS subsystem returns positional data along with fix quality and satellite count, allowing the system to assess whether the reading is reliable. Based on the combined BLE and GPS information, the handshake engine classifies the situation.

Examples include:

- the tag has genuinely moved out of range and the GPS position indicates that the user has left the area

- the BLE signal dropped due to interference, but the GPS position indicates the user is still within the expected area

- the GPS data is unreliable due to poor fix, in which case the event is ignored or deferred.

The handshake outcome leads to an internal state transition and the creation of a log entry

## Workflow Descriptions

In a departure scenario, the BLE signal is initially strong. As the user moves away, RSSI decreases until the tag is no longer detected. This will initiate an internal state transition, which will then trigger the handshake. If GPS confirms the user has left the defined geofence, the system records this as a departure event. In an arrival scenario, the device receives periodic GPS updates and detects that the coordinates fall within a predefined location. The system updates its state accordingly. If the BLE tag is still missing at this point, the system may classify this as an arrival without the tagged item.

## Design Constraints

Several constraints are expected at this stage. BLE RSSI is inherently noisy (Lee et al., 2025) meaning proximity readings may fluctuate even when the tag has not moved. GPS accuracy varies with weather conditions, satellite geometry, and environmental obstructions. The ESP32 must balance BLE scanning and GPS parsing within its available processing resources. Power consumption may increase if the scan interval or GPS polling frequency is set too aggressively. These are variables that will become evident when I begin building the artefact.

## Security Considerations

The system does not collect personal information and does not use any cloud or network components. BLE operates in unpaired advertising mode, which limits potential security concerns. Logged data consists only of device identifiers and environmental readings taken from the NMEA sentence.

# Data Requirements

## Data Types Stored

The system will store the following data to support analysis and evaluation:

- GPS latitude and longitude, representing the device’s absolute position

- GPS fix quality and satellite count, used to assess the reliability of each coordinate reading

- BLE RSSI values, representing the detected proximity of the tagged device

- Presence or absence of the BLE tag, derived from the BLE scan results

- Internal state transitions, generated by the BLE/GPS handshake logic

## Data Format

The data will be stored in a simple, structured format suitable for later analysis. The following is both appropriate and readable:

lat, lon,  fixQuality, satCount, rssi, eventType

This approach is human readable and allows the data to be imported easily into spreadsheets or analysis tools if it’s saved as a CSV file.

## Data Flow Summary

- BLE scanning produces RSSI values and presence information.

- GPS outputs positional data through NMEA sentences.

- The handshake engine evaluates both data sources and determines a classification.

- The logger records all relevant information to local storage.

# References:

# Appendix A: Report Structure Example

Project Technical Report

1) Project Overview

- Purpose and objectives of the software

- Scope of the project

- Assumptions, constraints and dependencies

- Definitions and acronyms

2) Requirements

Functional Requirements

- Detailed description of system functions and capabilities

- Input and output specifications

- Data processing operations

- Use cases, user stories (if applicable)

Non-Functional Requirements

- Performance requirements (e.g., response time, throughput)

- Security requirements

- Usability and accessibility standards

- Reliability and availability metrics

- Scalability considerations (if applicable)

3) System Architecture & Design

- Hardware and software requirements (including development and runtime environment)

- High-level system architecture

- Key components/functions/objects and their interactions

- Detailed description of system features, including workflow descriptions.

- Design and implementation constraints

- System qualities (e.g., maintainability, extensibility)

- Security protocols / considerations

- Testing strategy

4) Data Requirements

- Data models and schemas

- Data flow diagrams, class diagram or similar

5) Implementation

- Document the development process

- Code management, version control, etc. (evidence of following best practice)

- Discuss challenges, errors and limitations

6) Testing & Evaluation

- Testing overview / discussing testing requirements

- Functional Testing / Analysis (following testing strategy)

- Results/Evaluation Discussion

7) Conclusion

- Project conclusion/findings

- Limitations

- Future Work

# Appendix B:  Code Listing / GitHub Project Link